A residential building is located in Chennai which has total site area of 8,000 m2 and total built-up area is 14,000 m2.  Out of total built-up area 9,800 m2 is allocated for residential development and rest of the area will be used for commercial purpose.

The commercial area is operating for 6 day and 8 hours per day.

| Description | Residential Building | Commercial Building |
| --- | --- | --- |
| Built up area | 9,800 m2 | 4,200 m2 |
| GRIHA Benchmark | 70 | 90 (8 hours x 5 days) |
|  | The benchmark will be same | - For a day EPI (8h/day) = 90/5 =18 kWh/day/m2 - For six (6) days EPI (8h/7days) = 18x6=108 kWh/Year/m2 |
| Base Case Benchmark EPI (kWh/Year/m2) | 70×9,800+108×4,2009,800+4,200 Base case EPI =81.4 kWh/Year/m2 |  |